1 - Find the IP of the camera

2 - Browse the camera from IP, and set the camera login password. And static ip

3 - Login to the camera and expand the system option then click on safety option, uncheck the option. As shown in the image below

4 - expand the Network option then click on Access plateform option , and turn off ONVIF authentication

5 - Create an ONVIF user which is different from the login user password. And the user name should not be admin

Add Menual Camera

6 - Now enter ip, and in protocol select ONVIF, port 80, then enter ONVIF user password